2.0. Introduction

2.0.1 Why should I take this module?

🔹 Purpose: Configure switches & end devices for security & performance.
🔹 Why? Default settings exist, but customization is needed for your network.
🔹 Key Learning Points:

- Accessing Cisco IOS devices

- Basic configuration commands

- Configuring & verifying Cisco IOS devices

- Assigning IP addresses to end devices
🔹 Foundation: Essential first step before advanced network administration.

2.0.2 What will I learn to do in this module?

Module Title: Basic Switch and End Device Configuration

Module Objective: Implement initial settings including passwords, IP addressing, and default gateway parameters on a network switch and end devices.

2.1. Cisco IOS Access

2.1.1 Operating Systems

🔹 OS Components:

- Kernel: Directly interacts with hardware, manages resources.

- Shell: Interface for user interactions via CLI or GUI.

- Hardware: Physical components of the system.

🔹 User Interaction:

- CLI (Command-Line Interface): Text-based, low overhead, requires command knowledge.

- GUI (Graphical User Interface): Visual, user-friendly, but uses more system resources.

🔹 Example CLI Commands:

analyst@secOps ~]$ ls

- Lists files and directories.

🚀 Understanding OS structure is crucial for network and device management!

2.1.2 GUI

🔹 GUI (Graphical User Interface):

- User-friendly, uses icons, menus, and windows.

- Requires less technical knowledge.

- Can crash or lack advanced features.

🔹 CLI (Command-Line Interface):

- Used for network devices due to stability & efficiency.

- Requires command knowledge but offers full control.

🔹 Cisco Network Operating Systems:

- Cisco IOS: Used on Cisco routers & switches.

- Variants: IOS XE, IOS XR, NX-OS.

- Different devices use different IOS versions.

🔹 Home Routers:

- Use firmware as an OS.

- Typically configured via web-based GUI.

🚀 CLI mastery is essential for network management!

2.1.3 Purpose of an OS

🔹 PC Operating System (GUI-Based):

- Uses a mouse for selections & running programs.

- Allows text input & text-based commands.

- Displays output on a monitor.

🔹 Network OS (CLI-Based, e.g., Cisco IOS):

- Uses a keyboard to run CLI-based network programs.

- Requires text-based command entry.

- Displays output on a monitor.

🔹 Cisco IOS:

- Different devices use different IOS versions & feature sets.

- Default IOS can be upgraded for more features.

- Example: Cisco Catalyst 2960 Switch IOS versions.

🚀 Understanding IOS versions is key for network configuration & upgrades!

2.1.4 Access Methods